| Functional Requirements | Non-functional Requirements |
| --- | --- |
| The touch screen system will allow the user to interact with the display by touching the overlay. | Monitor display is still clear when the touch screen overlay is placed over it. |
| The touch screen system will indefinitely poll the interface by constantly toggling the emitting diodes in sequence and monitoring the sensors. | The presence of visible light has minimal effect on the operation of the touch screen. |
| The touch screen system will store the lines that in the case where a sensor does not detect an Infrared emitter. | Scalable: if more hardware is available, the hardware design and algorithms are easily adapted for more components. |
| The touch screen system will apply a cursor action on the Operating System when a valid interception is determined from 2 lines. | Portability: Attachable and detachable overlay. |
| The touch screen system will differentiate between different cursor actions, like dragging and clicking. | Minimal delay applying the action to the OS. |
|  | Minimal display coverage of 22 inches. |

Kiosk Supplementary Document

Table 1: Functional and non-functional Requirements for the touch screen system

Figure 1: All emitters ON, Effective touch area

TO

Figures 1.1: General Operation

Figure 1.2: Example of an Intercept

Figure 2: Bread board circuit of the emitter component

Figure 3: Bread board circuit of the sensor component

Figure 3.1: Touch screen overlay

Section A.1 Hysteresis add-on

Figure A.1: Hysteresis configuration with a voltage comparator

If a more consistent output voltage is needed that is less affected by the analog’s noise, a hysteresis can be used. A hysteresis in this sense means that the output state depends on the previous outputs. To implement this, 2 voltage thresholds are used to create a range where no change takes place.

To calculate the upper threshold, voltage divider is used:

Rh is connected to 5V due to Vout = 5V for Vin > Vth

VH = Vr x Ry(11Rx+1Rh)+Ry

VH = 2.7 V

Figure A.2: Upper Threshold visualization

To calculate the lower threshold, the same method is used:

Rh is connected to Vout = GND for Vin < Vth

VL = Vr x (11Ry+1Rh)Rx+(11Ry+1Rh)

VL = 2.3 V

Figure A.3: Lower Threshold visualization

Figure A.4: Output of a Comparator with Hysteresis Showing Single Transition [1]

In figure A.2 it presents that the output voltage transitions to logic HIGH, 5 V, when the input exceeds the upper threshold (VH) and it only transitions to logic LOW, 0 V, when the input is below the lower threshold (VL).

Figure 4: Activity Diagram of hardware control

Figure 5: Sequence Diagram of hardware control

Figure 6: State Diagram for hardware control

| Hardware testing methods | Software testing methods |
| --- | --- |
| 1. Continuity Tests of the circuits 2. Oscilloscope and voltmeter verification - Manual blocking | 1. Visual confirmation on hardware of correct sequence 2. Stub testing |

Table 2: Testing methods

References

[1] Texas Instruments, “TI Designs – Precision: Verified Design Comparator with Hysteresis Reference Design,” Texas Instruments, TIDU020A-May 2013-Revised June 2014. [Online]. Available: http://www.ti.com/lit/ug/tidu020a/tidu020a.pdf. [Accessed: March 10, 2018].